HIMMS and RSNA

IHE

Integrating the Healthcare Enterprise

Test Plan for Print Composers

2000-06-21

Marco Eichelberg1,  Klaus Kleber2, Jörg Riesmeier1

1  Kuratorium OFFIS e.V., Escherweg 2, 26121 Oldenburg, Germany

2  Institute for Microtherapy, Universitätsstraße 142, 44799 Bochum, Germany

Contents

1	Introduction	3

2	Print Composer Application Specification	4

2.1	Association Negotiation	4

2.1.1	Association Initiation Policy	4

2.1.2	Application Identification	4

2.1.3	Proposed Abstract Syntax Table	5

2.1.4	Proposed Transfer Syntax Table	6

2.1.5	Other Association Negotiation Items	6

2.2	Behavior of Printer SOP Class	7

2.3	Behavior of Basic Film Session SOP Class	8

2.4	Behavior of Basic Film Box SOP Class	10

2.5	Behavior of Basic Grayscale Image Box SOP Class	12

2.6	Behavior of Presentation LUT SOP Class	13

2.7	Behavior of other SOP Classes	15

2.8	General Comments	15

3	Test Procedure for Print Composers	16

3.1	Configuring and Using the DICOMscope Print SCP	16

3.2	Test Procedure	19

3.3	Syntactical Validation of DICOM Print Communication	21

3.4	Visual Validation of DICOM Print Communication	22

3.5	Open Issues of DICOM Print and Presentation LUT	22

# Introduction

This document is intended for those vendors participating in the IHE Year 2 demonstration which will supply one or more Print Composer Actors as defined in section 3.1 of the IHE Technical Framework (Year 2).

Print Composer Actors are required to implement the IHE “Print Request with Presentation LUT“ Transaction defined in section 6.23 of the Technical Framework. Interoperability between Print Composer and Print Server actors is a prerequisite for a successful performance of this transaction.  This document contains two main sections, both of which are intended to facilitate interoperability.

Section 2 contains an Application Specification Form which allows to describe the behavior of a Print Composer in terms of DICOM services, messages and objects.  It is roughly structured like a DICOM Conformance Statement but goes into more detail than a typical conformance statement.

This form should be filled in by all vendors supplying a Print Composer.  The filled-in forms will be made available to vendors supplying Print Server Actors. This will allow to identify certain DICOM network protocol related problems at an early point in time.

If a vendor supplies more than one Print Composer, or if the Print Composer can be operated in different modes which exhibit different behavior on the DICOM network protocol level, multiple copies of the form should be filled in.  Please send the completed forms to Steve Moore at the Mallinckrodt Institute of Radiology:

email:	moores@mir.wustl.edu

fax: 314-362-6971

phone: 314-362-6965

If you send the forms as Word attachments, please take the time to run a virus scanner on the document before you send it. The forms should be completed and returned to Steve Moore by INSERT_DEADLINE_HERE.

If a DICOM conformance statement is available for the Print Composer, it would be useful if the conformance statement is provided together with (but not instead of) the filled-in specification form.

Section 3 contains a test procedure describing how Print Composers can be tested with the IHE Year 2 MESA tools.  The MESA tools contain a Print Server that implements the requirements of the IHE Year 2 “Print Request with Presentation LUT” transaction.  It should be noted that interoperability between a Print Composer and the MESA testing Print Server does improve the probability for interoperability between the Print Composer Actor and a Print Server Actor provided by a different vendor, but is in no way a guarantee for interoperability.

IHE Year 2 Print Composer Actors

Application Specification Form

# Print Composer Application Specification

Application vendor

Name and software Version of the application

Person filling in this form, place, date

## Association Negotiation

This section describes the association negotiation behavior of the application.

### Association Initiation Policy

Please describe the real world activity leading to an association initiation by the application.

Association initiation policy

### Application Identification

Implementation Class UID

Implementation Version Name

Calling Application Entity Title

Max PDU Size

General Comments on the Application:

Comments

### Proposed Abstract Syntax Table

Please check the SOP Classes / Meta SOP Classes that are proposed by the application during the association negotiation (A-ASSOCIATE-RQ):

| Yes | SOP Class Name | SOP Class UID |
| --- | --- | --- |
|  | Basic Grayscale Print Management (Meta) | 1.2.840.10008.5.1.1.9 |
|  | Basic Color Print Management (Meta) | 1.2.840.10008.5.1.1.18 |
|  | Pull Stored Print Management (Meta) | 1.2.840.10008.5.1.1.32 |
|  | Presentation LUT | 1.2.840.10008.5.1.1.23 |
|  | Basic Annotation Box | 1.2.840.10008.5.1.1.15 |
|  | Basic Print Image Overlay Box | 1.2.840.10008.5.1.1.24.1 |
|  | Print Job | 1.2.840.10008.5.1.1.14 |
|  | Print Queue Management | 1.2.840.10008.5.1.1.26 |
|  | Printer Configuration Retrieval | 1.2.840.10008.5.1.1.16.376 |
|  | Verification | 1.2.840.10008.1.1 |
|  | Other (please specify): |  |

General Comments on the Proposed Abstract Syntax Table:

Comments

### Proposed Transfer Syntax Table

Please check the Transfer Syntaxes that are proposed by the application during the association negotiation (A-ASSOCIATE-RQ) for each Abstract Syntax:

| Yes | Transfer Syntax Name | Transfer Syntax UID |
| --- | --- | --- |
|  | Implicit VR Little Endian | 1.2.840.10008.1.2 |
|  | Explicit VR Little Endian | 1.2.840.10008.1.2.1 |
|  | Explicit VR Big Endian | 1.2.840.10008.1.2.2 |
|  | JPEG Baseline (Process 1) | 1.2.840.10008.1.2.4.50 |
|  | JPEG Lossless (Process 14) Selection Value 1 | 1.2.840.10008.1.2.4.70 |
|  | RLE Lossless | 1.2.840.10008.1.2.5 |
|  | Other (please specify): |  |

General Comments on the Proposed Transfer Syntax Table:

Comments

### Other Association Negotiation Items

| Yes | Application Context |  |
| --- | --- | --- |
|  | DICOM Standard Application Context | 1.2.840.10008.3.1.1.1 |

If the application supports asynchronous operation, please specify the Maximum-number-operations-invoked and Maximum-number-operations-performed values sent during association negotiation.

Asynchronous Operations Window

If the application supports SCU/SCP role selection, please specify which roles are proposed for which SOP class.

SCU/SCP Role Selection

If the application supports extended negotiation for any of the proposed SOP classes, please specify.

Extended Negotiation

General Comments on the Association Negotiation Items:

Comments

## Behavior of Printer SOP Class

Please specify which operations may be invoked or received for the Printer SOP Class

| Yes | Operation | Type |
| --- | --- | --- |
|  | N-GET | Request |
|  | N-EVENT-REPORT | Indication |

Please specify which attributes may be requested within an N-GET operation.

|  | Empty N-GET attribute identifier list requesting all attributes, or: |  |
| --- | --- | --- |
| Yes | Attribute Name | Tag |
|  | Printer Status | (2110,0010) |
|  | Printer Status Info | (2110,0020) |
|  | Printer Name | (2110,0030) |
|  | Manufacturer | (0008,0070) |
|  | Manufacturer Model Name | (0008,1090) |
|  | Device Serial Number | (0018,1000) |
|  | Software Versions | (0018,1020) |
|  | Date Last Calibration | (0018,1200) |
|  | Last Calibration | (0018,1201) |
|  | Other (please specify): |  |

Please specify whether the application can correctly handle the receipt of an N-EVENT-REPORT-RQ message after sending a different request (e. g. N-CREATE-RQ) and before receiving the response to this request (e. g. N-CREATE-RSP):

| Yes | Application Behavior |
| --- | --- |
|  | Application is known to handle this case correctly |
|  | Application is known not to handle this case |
|  | Application behavior is not known |

General Comments on the Printer SOP Class:

Comments

## Behavior of Basic Film Session SOP Class

Please specify which operations may be invoked for the Basic Film Session SOP Class

| Yes | Operation | Type |
| --- | --- | --- |
|  | N-CREATE | Request |
|  | N-DELETE | Request |
|  | N-SET | Request |
|  | N-ACTION | Request |

Please specify which attributes and attribute values may be sent as part of an N-CREATE request:

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Number of Copies | (2000,0010) | IS |  |
|  | Print Priority | (2000,0020) | CS |  |
|  | Medium Type | (2000,0030) | CS |  |
|  | Film Destination | (2000,0040) | CS |  |
|  | Illumination | (2010,015E) | US |  |
|  | Reflected Ambient Light | (2010,0160) | US |  |
|  | Referenced Presentation LUT Sequence | (2050,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Film Session Label | (2000,0050) | LO |  |
|  | Memory Allocation | (2000,0060) | IS |  |
|  | Owner ID | (2100,0160) | SH |  |
|  | Proposed Study Sequence | (2130,00A0) | SQ | (attributes within Proposed Study SQ not shown in this list) |
|  | Other (please specify): |  |  |  |

Please specify which attributes and attribute values may be sent as part of an N-SET request:

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Number of Copies | (2000,0010) | IS |  |
|  | Print Priority | (2000,0020) | CS |  |
|  | Medium Type | (2000,0030) | CS |  |
|  | Film Destination | (2000,0040) | CS |  |
|  | Illumination | (2010,015E) | US |  |
|  | Reflected Ambient Light | (2010,0160) | US |  |
|  | Referenced Presentation LUT Sequence | (2050,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Film Session Label | (2000,0050) | LO |  |
|  | Memory Allocation | (2000,0060) | IS |  |
|  | Owner ID | (2100,0160) | SH |  |
|  | Proposed Study Sequence | (2130,00A0) | SQ | (attributes within Proposed Study SQ not shown in this list) |
|  | Other (please specify): |  |  |  |

General Comments on the Basic Film Session SOP Class:

Comments

## Behavior of Basic Film Box SOP Class

Please specify which operations may be invoked for the Basic Film Box SOP Class

| Yes | Operation | Type |
| --- | --- | --- |
|  | N-CREATE | Request |
|  | N-DELETE | Request |
|  | N-SET | Request |
|  | N-ACTION | Request |

Please specify which attributes and attribute values may be sent as part of an N-CREATE request:

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Image Display Format | (2010,0010) | ST |  |
|  | Referenced Film Session Sequence | (2010,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Film Orientation | (2010,0040) | CS |  |
|  | Film Size ID | (2010,0050) | CS |  |
|  | Magnification Type | (2010,0060) | CS |  |
|  | Max Density | (2010,0130) | US |  |
|  | Configuration Information | (2010,0150) | ST |  |
|  | Referenced Presentation LUT Sequence | (2050,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Annotation Display Format ID | (2010,0030) | CS |  |
|  | Smoothing Type | (2010,0080) | CS |  |
|  | Border Density | (2010,0100) | CS |  |
|  | Empty Image Density | (2010,0110) | CS |  |
|  | Min Density | (2010,0120) | US |  |
|  | Trim | (2010,0140) | CS |  |
|  | Requested Resolution ID | (2020,0050) | CS |  |
|  | Illumination | (2010,015E) | US |  |
|  | Reflected Ambient Light | (2010,0160) | US |  |
|  | Other (please specify): |  |  |  |

Please specify which attributes and attribute values may be sent as part of an N-SET request:

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Magnification Type | (2010,0060) | CS |  |
|  | Max Density | (2010,0130) | UI |  |
|  | Configuration Information | (2010,0150) | ST |  |
|  | Referenced Presentation LUT Sequence | (2050,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Smoothing Type | (2010,0080) | CS |  |
|  | Border Density | (2010,0100) | CS |  |
|  | Empty Image Density | (2010,0110) | CS |  |
|  | Min Density | (2010,0120) | US |  |
|  | Trim | (2010,0140) | CS |  |
|  | Illumination | (2010,015E) | US |  |
|  | Reflected Ambient Light | (2010,0160) | US |  |
|  | Other (please specify): |  |  |  |

General Comments on the Basic Film Box SOP Class:

Comments

## Behavior of Basic Grayscale Image Box SOP Class

Please specify which operations may be invoked for the Basic Film Box SOP Class

| Yes | Operation | Type |
| --- | --- | --- |
|  | N-SET | Request |

Please specify which attributes and attribute values may be sent as part of an N-SET request:

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Image Position | (2020,0010) | US |  |
|  | Basic Grayscale Image Sequence | (2020,0110) | SQ |  |
|  | >Samples Per Pixel | (0028,0002) | US |  |
|  | >Photometric Interpretation | (0028,0004) | CS |  |
|  | >Rows | (0028,0010) | US |  |
|  | >Columns | (0028,0011) | US |  |
|  | >Pixel Aspect Ratio | (0028,0034) | IS |  |
|  | >Bits Allocated | (0028,0100) | US |  |
|  | >Bits Stored | (0028,0101) | US |  |
|  | >High Bit | (0028,0102) | US |  |
|  | >Pixel Representation | (0028,0103) | US |  |
|  | >Pixel Data | (7FE0,0010) | OB/OW |  |
|  | Polarity | (2020,0020) | CS |  |
|  | Referenced Image Overlay Box Sequence | (2020,0130) | SQ |  |
|  | >SOP Class UID | (0008,1150) | UI |  |
|  | >SOP Instance UID | (0008,1155) | UI |  |
|  | Magnification Type | (2010,0060) | CS |  |
|  | Smoothing Type | (2010,0080) | CS |  |
|  | Max Density | (2010,0130) | US |  |
|  | Min Density | (2010,0120) | US |  |
|  | Configuration Information | (2010,0150) | ST |  |
|  | Requested Image Size | (2020,0030) | DS |  |
|  | Requested Decimate/Crop Behavior | (2020,0040) | CS |  |
|  | Referenced Presentation LUT Sequence | (2050,0500) | SQ |  |
|  | >Referenced SOP Class UID | (0008,1150) | UI |  |
|  | >Referenced SOP Instance UID | (0008,1155) | UI |  |
|  | Original Image Sequence | (2130,00C0) | SQ | (attributes within Original Image SQ not shown in this list) |
|  | Other (please specify): |  |  |  |

General Comments on the Basic Grayscale Image Box SOP Class:

Comments

## Behavior of Presentation LUT SOP Class

Please specify which operations may be invoked for the Basic Film Box SOP Class

| Yes | Operation | Type |
| --- | --- | --- |
|  | N-CREATE | Request |
|  | N-ACTION | Request |

Please specify which attributes and attribute values may be sent as part of an N-SET request. Note: the content of the LUT Descriptor is quite essential to determine whether a Print SCP will be able to process a Presentation LUT Sequence. All possible combinations of the three LUT descriptor values should be specified.

| Yes | Attribute Name | Tag | VR | Attribute Value(s) |
| --- | --- | --- | --- | --- |
|  | Presentation LUT Sequence | (2050,0010) | SQ |  |
|  | >LUT Descriptor | (0028,3002) | US/ SS |  |
|  | >LUT Explanation | (0028,3003) | LO |  |
|  | >LUT Data | (0028,3006) | OW/ US/ SS |  |
|  | Presentation LUT Shape | (2050,0020) | CS |  |
|  | Other (please specify): |  |  |  |

General Comments on the Presentation LUT SOP Class:

Comments

## Behavior of other SOP Classes

Please describe behavior of other proposed SOP classes.

Behavior of other Proposed SOP Classes

## General Comments

Please fill in general comments to the “Print Composer Test”.

General Comments

# Test Procedure for Print Composers

The IHE Year 2 MESA Tools contain, among other components, a Java-based GUI application called “DICOMscope”.  This application is used in the test procedure for Print Composers.  Installation instructions and a user manual for the DICOMscope application are provided in different documents with the IHE Year 2 MESA distribution.  The following discussion assumes that the application has been successfully installed and that the user has a basic understanding of the application’s concepts as laid out in the user manual.  However, not all functions offered by the DICOMscope application are required or used in this test procedure.

## Configuring and Using the DICOMscope Print SCP

The DICOMscope Print SCP shares a common configuration file with other parts of the DICOMscope application.  The configuration file is a simple text file and can be edited with any text editor.  The overall structure of the configuration file is described in the DICOMscope manual.  Of interest for the Print SCP are only the parts of the file that are started with the identifier [[COMMUNICATION]].

DICOMscope allows to have multiple Print SCPs operating in parallel, with different configurations. Each configuration is described by a separate section, which is started by a section name in square brackets that must be unique for the configuration file, e. g. [MY_PRINTER].  Section names should consist only of uppercase letters, numbers and the underscore character.  The section name is followed by a number of entries of the form keyword = value.  The following table describes all keywords that can be used to configure a DICOMscope Print SCP instance.

| Keyword | Value | Description |
| --- | --- | --- |
| Description | string | Human readable description of the Print SCP. |
| Hostname | string | The IP number or domain name of the communication partner in conventional TCP/IP notation. Value must be: localhost |
| Port | 1..65535 | IP port number under which the Print SCP receives DICOM associations. Different Print SCPs on one system require different port numbers. Please note that port numbers < 1024 require root privileges on Unix systems. |
| Type | string | Value must be: LOCALPRINTER |
| AETitle | string | DICOM application entity title used by the Print SCP to identify itself during association negotiation. |
| MaxPDU | 4096..131072 | Maximum receive PDU (protocol data unit) size to use. Must be between 4096 and 131072. Default is 16384. |
| DisableNewVRs | true or false | Do not create data types (VRs) that were defined in post-1993 editions of the DICOM standard when converting implicit VR files to explicit VR network transmission. Optional setting, default is: false. |
| ImplicitOnly | true or false | Only negotiate the default Implicit VR Little Endian transfer syntax for all abstract syntaxes. This setting is useful if we're communicating with very old DICOM software which claims to support Explicit VR communication but fails to do so... |
| Supports12Bit | true or false | Accept image pixel data with 12 bits/pixel. Optional setting, default is: true. |
| SupportsPresentationLUT | true or false | Activate support for the Presentation LUT SOP Class. Optional setting, default is: false. |
| PresentationLUTinFilmSession | true or false | The 1999 edition of the DICOM standard contains an inconsistency in the definition of the Presentation LUT SOP class. The attributes "Referenced Presentation LUT", "Illumination" and "Reflected Ambient Light" can either be part of the Basic Film Session or be part of the Basic Film Box. DICOM Correction Proposal CP 173 defines that these attributes have to be specified on Basic Film Box level. However, not all existing Print SCPs supporting the Presentation LUT SOP Class are implemented in accordance with CP 173. The Print SCP can, therefore, be configured to use either Film Session or Film Box (but never both). If flag is true, attributes are accepted and returned in Basic Film Session instead of Basic Film Box. Optional setting, default is: false (which means that behavior is consistent with CP 173.) |
| PresentationLUTMatchRequired | true or false | Some Print SCPs which support Presentation LUTs require that the number of entries in a Presentation LUT matches the bit depth of the image pixel data (4096 entries for 12 bit pixel data, 256 entries for 8 bit pixel data). If flag is true, enforce a matching rule as described above. All Presentation LUT N-CREATE or Basic Grayscale Image Box N-SET operations that would violate the rule will be refused. Optional setting, default is: true. |
| SupportsImageSize | true or false | Some Print SCPs do not support the optional "Requested Image Size" attribute in Basic Grayscale Image Box N-SET operations. If flag is true, support requested image size. Otherwise refuse N-SET operations containing a requested image size attribute. Optional setting, default is: false. |
| SupportsDecimateCrop | true or false | Some Print SCPs do not support the optional "Requested Decimate/Crop Behavior" attribute defined in Supplement 37 in Basic Grayscale Image Box N-SET operations. If flag is true, support requested decimate/crop behavior. Otherwise refuse N-SET operations containing this attribute. Optional setting, default is: false. |
| SupportsTrim | true or false | Some Print SCPs do not support the optional "Trim" attribute in Basic Film Box N-CREATE or N-SET operations. If flag is true, support trim. Otherwise refuse N-CREATE or N-SET operations containing this attribute. Optional setting, default is: false. |
| DisplayFormat | string\string… | this entry specifies all STANDARD\C,R image display formats accepted by the Print SCP for portrait orientation. Landscape image display formats are derived automatically. This is a mandatory entry, no default. |
| FilmSizeID | string\string… | Film Size ID identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory setting. |
| MediumType | string\string… | Medium Type identifiers supported by the printer for the Basic Film Session. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| ResolutionID | string\string… | Requested Resolution ID identifiers supported by the printer for the Basic Film Box (optional attribute defined in Supplement 37). Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| MagnificationType | string\string… | Magnification Type identifiers supported by the printer for the Basic Film Box or Basic Grayscale Image Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| SmoothingType | string\string… | Smoothing Type identifiers supported by the printer for the Basic Film Box or Image Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| BorderDensity | string\string… | Border Density identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. If any of the identifiers is numeric, then all numbers are accepted. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| EmptyImageDensity | string\string… | Empty Image Density identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. If any of the identifiers is numeric, then all numbers are accepted. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| MaxDensity | int | Default max density that is used when the Print SCU does not specify max density. This is a mandatory entry. Note: the Print SCP will accept any syntactically correct value for Max Density even if the value is higher than the Max Density default defined here. |
| MinDensity | int | Default min density that is used when the Print SCU does not specify min density. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. Note: the Print SCP will accept any syntactically correct value for Min Density even if the value is lower than the Min Density default defined here. |
| Configuration_n | string | Configuration Information that can be sent to the printer for the Basic Film Box or Image Box. Only a single value per entry can be specified because values may contain backslash characters. (VR=ST). Keywords are "Configuration_1", "Configuration_2", etc., without leading zeroes. These are optional settings. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. The Print SCP default for Configuration Information is always an empty string. |
| FilmDestination | string\string… | Film Destination identifiers supported by the Print SCP for the Basic Film Session. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| OmitSOPClassUIDFromCreateResponse | true or false | The Affected SOP Class UID attribute is optional in DIMSE N-CREATE-RSP messages but some clients rely on its presence. This setting can be used to test client behavior. Defines whether the Print SCP should omit Affected SOP Class UID in DIMSE N-CREATE-RSP messages. This is an optional setting, default is false. |

When the DICOMscope application is started, a separate Print SCP application is started for each section in the configuration file correctly defining a Print SCP.  As an alternative, a Print SCP can also be started manually without the DICOMscope GUI.  The command line Print SCP is called “dcmprscp” and is automatically installed together with DICOMscope.  The following command line options are available:

The name of the configuration file (option –-config) and the printer name (the section heading from the configuration file, option –-printer) should always be passed on the command line.  If the tool is started with the verbose and dump options, a detailed log of it’s operations including a complete log of the DIMSE communication is written to the console.  The –-logfile option causes this information to be written into a text file in the log subdirectory for later examination.

dcmprscp: DICOM basic grayscale print management SCP

usage: dcmprscp [options]

general options:

-h  --help          print this help text and exit

-v  --verbose       verbose mode, print actions

-d  --debug         debug mode, print debug information

processing options:

-c  --config        [f]ilename: string

process using settings from configuration file

-p  --printer       [n]ame: string (default: 1st printer in cfg file)

select printer with identifier [n] from cfg file

+d  --dump          print all DIMSE messages

-l  --logfile       print to log file instead of stdout

When running, the Print SCP accepts a single DICOM association at a time – multiple clients in parallel are not supported.  Since Print SCP is not connected to a real printer, it does not create hardcopies.  Instead, a print job is stored in the local database (which is shared with the other parts of the DICOMscope application).  Each film of a print job is stored as a DICOM Stored Print object.  Each image of a print job is stored as a DICOM Hardcopy Grayscale Image object.  When Print SCP is run in parallel with the DICOMscope application, the DICOMscope database browser will indicate that new objects have arrived and will allow to load and display these objects.  Hardcopy Grayscale images can be loaded and displayed as any other DICOM image, however, specific characteristics like a Presentation LUT or the image’s polarity are not preserved in the Hardcopy Grayscale image but in the Stored Print object.  Loading a Stored Print causes a preview of the hardcopy to be rendered in DICOMscope’s print preview panel.  This display correctly renders Presentation LUT, Polarity etc.

## Test Procedure

The IHE Year 2 “Print Request with Presentation LUT“ transaction (section 6.23 of the Technical Framework) defines minimum requirements for Print Composer actors.  In particular, the Presentation LUT SOP Class must be supported, and Presentation LUTs must be referenced on Basic Film Box level (in accordance with DICOM Correction Proposal 173).  However, Print Composers still have many “degrees of freedom” in terms of their support for optional attributes.  Therefore, we do not propose a static set of tests for a Print Composer, but instead a (rather general) procedure.  The DICOMscope software default installation defines two Print SCPs:

- PRINTSCP1 accepts associations on port 10005.  This Print SCP supports most optional features of the DICOM print protocol:

- Image display formats 1x1, 1x2, 2x2, 2x3, 3x3, 3x4, 3x5, 4x4 and 4x5 are supported for Portrait orientation, and 1x1, 2x1 etc. for Landscape,

- All standard defined terms for Border Density, Empty Image Density, Film Destination, Film Size ID, Magnification Type, Medium Type and Requested Resolution ID are supported,

- Min Density, Trim, Requested Image Size and Requested Decimate/Crop Behavior are supported,

- Smoothing Type is supported (value: NONE),

- Images can be transmitted with either 8 or 12 bits per pixel,

- The Affected SOP Class UID is always included in N-CREATE-RSP messages,

- Presentation LUTs with a number of entries other than 256 or 4096 are accepted.

- PRINTSCP2 accepts associations on port 10006.  This Print SCP is restricted to the minimum of features required by the IHE technical framework:

- Image display formats must be 1x1

- Border Density, Empty Image Density, Requested Resolution ID, Min Density, Smoothing Type, Trim, Requested Image Size and Requested Decimate/Crop Behavior are not supported,

- None of the standard defined terms for Film Destination and Medium Type are supported

- Only the required terms for Film Size ID and Magnification Type defined in the IHE Technical Framework are supported,

- Images can be transmitted with either 8 or 12 bits per pixel,

- The Affected SOP Class UID is never included in N-CREATE-RSP messages,

- Presentation LUTs with a number of entries other than 256 or 4096 are rejected. Presentation LUTs with 256 entries can only be used with 8 bit/pixel images, LUTs with 4096 entries can only be used with 12 bit/pixel images.

- Only the default Implicit VR Little Endian Transfer Syntax is negotiated.

Testing a Print Composer should start with PRINTSCP1.  Vendors should create a couple of print jobs exercising the different options available to their Print Composer implementations.  The results of these print jobs should be checked as described in the following two sections.

If the print jobs succeed, the tests should be repeated with PRINTSCP2, which poses many restrictions on the Print Composer.  If printing with PRINTSCP2 also succeeds, probability is high that printing on other Print Server actors will also succeed.

A third, optional series of tests can be performed by changing the following setting in the DICOMscope configuration file for PRINTSCP2 and repeating the tests with PRINTSCP2:

Supports12Bit = false

The DICOMscope software must be restarted after this change.  Now PRINTSCP2 will refuse any image transmitted in a Basic Grayscale Image Box with more than 8 bits/pixel.  This is the most restrictive setting available with the DICOMscope Print SCP.

As described above, all print jobs created by the DICOMscope Print SCP are stored in the local database as a number of Stored Print and Hardcopy Grayscale Image objects.  With a large number of tests, the database can quickly become crowded.  We recommend to erase the database before starting a new series of tests.  This can be done by terminating the DICOMscope application and deleting all files (including index.dat) from the subdirectory named database in the DICOMscope folder.  We also recommend to delete all old log files from the subdirectory named log before starting a new series of tests.

## Syntactical Validation of DICOM Print Communication

The DICOMscope Print SCP creates detailed logs of all DICOM communication with a Print SCU.  The log files are in plain text format and can be opened and printed with any text editor.  They are located in the log subdirectory of the DICOMscope application folder.  The filenames have the form

PrintSCP_XXXX_YYYYMMDD_HHMMSS.log

where XXXX is the symbolic printer name (section title from the DICOMscope configuration file) and YYYYMMDD HHMMSS are date and time when the application was started.  A log file may contain multiple print jobs since it remains open as long as the DICOMscope Print SCP is running.

For each print job, the log file shows the DICOM association negotiation first, followed by the DIMSE messages exchanged between SCU and SCP.  Figure 3.3.1 below shows an example for an association request.  All items of the A-ASSOCIATE-RQ protocol data unit as defined in part 7 and 8 of the DICOM standard are shown.  A basic understanding of the DICOM association protocol (ACSE) is required to read this part of the log.  When evaluating the log, it should be checked whether the Presentation LUT SOP class has been both proposed and accepted.

Figure 3.3.1: Print SCP Log – Association Negotiation

====================== BEGIN A-ASSOCIATE-RQ =====================

Our Implementation Class UID:    1.2.276.0.7230010.3.0.3.4.1

Our Implementation Version Name: OFFIS_DCMTK_341

Their Implementation Class UID:    1.2.276.0.7230010.3.0.3.4.1

Their Implementation Version Name: OFFIS_DCMTK_341

Application Context Name:    1.2.840.10008.3.1.1.1

Calling Application Name:    DCMPSTATE

Called Application Name:     LOCALPRINTER_1

Responding Application Name:

Our Max PDU Receive Size: 32768

Their Max PDU Receive Size: 32768

Presentation Contexts:

Context ID:        1 (Proposed)

Abstract Syntax: =BasicGrayscalePrintManagementMetaSOPClass

Proposed SCP/SCU Role: Default

Accepted SCP/SCU Role: Default

Proposed Transfer Syntax(es):

=BigEndianExplicit

=LittleEndianExplicit

=LittleEndianImplicit

Context ID:        3 (Proposed)

Abstract Syntax: =PresentationLUTSOPClass

Proposed SCP/SCU Role: Default

Accepted SCP/SCU Role: Default

Proposed Transfer Syntax(es):

=BigEndianExplicit

=LittleEndianExplicit

=LittleEndianImplicit

Requested Extended Negotiation: none

Accepted Extended Negotiation: none

======================= END A-ASSOCIATE-RQ ======================

Figure 3.3.2 below shows an example for the DIMSE message log. It shows a N-SET request sent to a Basic Grayscale Image Box object, followed by the N-SET response from the Print SCP.  Evaluating this part of the log requires a basic understanding of the DICOM Message Service Element (DIMSE) and the definition of the DICOM Print Management Service Class.  In particular it should be checked whether a Presentation LUT instance has been created by the Print Composer and whether it has been correctly referenced (activated) as part of the Basic Film Box N-CREATE request.  The status codes sent back by the Print SCP for each message should be checked.  For most error and warning codes returned by the Print SCP, additional information is stored in the log file (preceding the response message).

Figure 3.3.2: Print SCP Log – DIMSE Communication

===================== INCOMING DIMSE MESSAGE ====================

Message Type                  : N-SET RQ

Message ID                    : 5

Requested SOP Class UID       : BasicGrayscaleImageBoxSOPClass

Requested SOP Instance UID    : 1.2.276.0.7230010.3.4.2139363186.2206.960455839.7

Data Set                      : present

# Dicom-Data-Set

# Used TransferSyntax: BigEndianExplicit

(2020,0010) US    1                                      #     2,  1  ImageBoxPosition

(2020,0110) SQ    (Sequence with explicit Length #=1)    #2097262,  1  BasicGrayscaleImageSequence

(fffe,e000) na    (Item with explicit Length  #=9)       #2097254,  1  Item

(0028,0002) US    1                                      #     2,  1  SamplesPerPixel

(0028,0004) CS    [MONOCHROME2]                          #    12,  1  PhotometricInterpretation

(0028,0010) US    1024                                   #     2,  1  Rows

(0028,0011) US    1024                                   #     2,  1  Columns

(0028,0100) US    16                                     #     2,  1  BitsAllocated

(0028,0101) US    12                                     #     2,  1  BitsStored

(0028,0102) US    11                                     #     2,  1  HighBit

(0028,0103) US    0                                      #     2,  1  PixelRepresentation

(7fe0,0010) OW    0160\0160\0160\0160\0177\0177\0177\0177\017d\017d... #2097152,  1  PixelData

(fffe,e00d) na    (ItemDelimitationItem for re-encoding) #     0,  1  ItemDelimitationItem

(fffe,e0dd) na    (SequenceDelimitationItem for re-enc.) #     0,  1  SequenceDelimitationItem

======================= END DIMSE MESSAGE =======================

===================== OUTGOING DIMSE MESSAGE ====================

Message Type                  : N-SET RSP

Message ID Being Responded To : 5

Affected SOP Class UID        : none

Affected SOP Instance UID     : none

Data Set                      : present

DIMSE Status                  : 0x0000: Success

# Dicom-Data-Set

# Used TransferSyntax: UnknownTransferSyntax

(2020,0010) US    1                                      #     2,  1  ImageBoxPosition

======================= END DIMSE MESSAGE =======================

## Visual Validation of DICOM Print Communication

The second step of a print job validation is the visual test.  The stored print object related to the print job should be located in DICOMscope’s database browser and should be loaded into DICOMscope’s Print Preview panel (e. g. by double-clicking on the Stored Print object).  The Print Preview should display a correct softcopy look-alike of the intended hardcopy.  In particular, the images on the Print Preview are displayed with the appropriate Presentation LUT applied.  If DICOMscope is calibrated for the current softcopy display, GSDF is also correctly applied.  It should be noted that certain optional attributes such as Magnification Type, Smoothing Type, Requested Image Size and Requested Decimate/Crop Behavior are ignored in the Print Preview.  Scaling of the images is always done with a bilinear interpolation.  The image data, polarity, Presentation LUT, Image Display Format and Orientation are observed, however.

## Open Issues of DICOM Print and Presentation LUT

This section describes a number of known issues with the DICOM Print Management Service Class that Print Composer vendors should be aware of when testing their implementations.

- The definition of the Presentation LUT SOP Class does not restrict the number of entries of a Presentation LUT, nor the “First Value Mapped” entry of the LUT descriptor.  However, Presentation LUTs with a number of entries other than 256 or 4096 do not seem to be too useful.  Therefore, Print Server vendors might choose not to accept such Presentation LUTs.  Since the definition of the Presentation LUT SOP Class does not define any LUTs that must be supported by all Print Servers (except the LUT Shape constants), this is not a violation of the standard.

- The Basic Grayscale Print Meta SOP Class allows image data to be transmitted either with 8 bits/pixel (Bits Stored) or 12 bits/pixel.  There is no requirement, though, that a Print Server must support both.  Vendors might choose to support only one or the other variant.

- The concept of a Presentation LUT is used for two different purposes in the DICOM standard.  On one hand, there is the Presentation LUT SOP Class defined as an extension of the Print Management Service Class.  On the other hand, there is the Softcopy Grayscale Presentation State SOP Class (Supplement 33) which also makes intensive use of Presentation LUTs.  Both SOP Classes make use of the same DICOM attributes and sequences to represent Presentation LUTs.  However, the semantics of how to apply a Presentation LUT to image data are different!  In softcopy, a Presentation LUT with 256 entries can be applied to an image with 12 bits/pixel and result in a correct image display.  If the same Presentation LUT was applied to the same image as part of the hardcopy Presentation LUT SOP Class, the result would look almost completely white.  This is an inconsistency in the standard, not in it’s implementation.