HIMMS and RSNA

IHE

Integrating the Healthcare Enterprise

Test Plan for Print Composers

2003-12-01

Marco Eichelberg1,  Klaus Kleber2, Jörg Riesmeier1

1  Kuratorium OFFIS e.V., Escherweg 2, 26121 Oldenburg, Germany

2  VISUS Technology Transfer GmbH, Philippstraße 5, D-44803 Bochum, Germany

Contents

1	Introduction	3

2	Test Procedure for Print Composers	3

2.1	Configuring and Using the DICOMscope Print SCP	3

2.2	Test Procedure	7

2.3	Syntactical Validation of DICOM Print Communication	8

2.4	Visual Validation of DICOM Print Communication	10

2.5	Open Issues of DICOM Print and Presentation LUT	10

# Introduction

This document is intended for those vendors participating in IHE which supply Print Composer actors for the “Consistent Presentation of Images” Integration Profile.  Print Composer Actors are required to implement the IHE Transaction 23 “Print Request with Presentation LUT” of the IHE Radiology Technical Framework

This document contains a test procedure describing how Print Composers can be tested with the IHE MESA tools.  The MESA tools contain a Print Server that implements the requirements of the IHE“Print Request with Presentation LUT” transaction.  It should be noted that interoperability between a Print Composer and the MESA testing Print Server does improve the probability of interoperability between the Print Composer Actor and a Print Server Actor provided by a different vendor, but is in no way a guarantee for interoperability.

# Test Procedure for Print Composers

The IHE MESA Tools contain, among other components, a Java-based GUI application called “DICOMscope”.  This application is used in the test procedure for Print Composers.  Installation instructions and a user manual for the DICOMscope application are provided in different documents with the IHE MESA distribution.  The following discussion assumes that the application has been successfully installed and that the user has a basic understanding of the application’s concepts as laid out in the user manual.  However, not all functions offered by the DICOMscope application are required or used in this test procedure.

## Configuring and Using the DICOMscope Print SCP

The DICOMscope Print SCP shares a common configuration file with other parts of the DICOMscope application.  The configuration file is a simple text file and can be edited with any text editor.  The overall structure of the configuration file is described in the DICOMscope manual.  Of interest for the Print SCP are only the parts of the file that are started with the identifier [[COMMUNICATION]].

DICOMscope allows to have multiple Print SCPs operating in parallel, with different configurations. Each configuration is described by a separate section, which is started by a section name in square brackets that must be unique for the configuration file, e. g. [MY_PRINTER].  Section names should consist only of uppercase letters, numbers and the underscore character.  The section name is followed by a number of entries of the form keyword = value.  The following table describes all keywords that can be used to configure a DICOMscope Print SCP instance.

| Keyword | Value | Description |
| --- | --- | --- |
| Description | string | Human readable description of the Print SCP. |
| Hostname | string | The IP number or domain name of the communication partner in conventional TCP/IP notation. Value must be: localhost |
| Port | 1..65535 | IP port number under which the Print SCP receives DICOM associations. Different Print SCPs on one system require different port numbers. Please note that port numbers < 1024 require root privileges on Unix systems. |
| Type | string | Value must be: LOCALPRINTER |

| AETitle | string | DICOM application entity title used by the Print SCP to identify itself during association negotiation. |
| --- | --- | --- |
| MaxPDU | 4096..131072 | Maximum receive PDU (protocol data unit) size to use. Must be between 4096 and 131072. Default is 16384. |
| DisableNewVRs | true or false | Do not create data types (VRs) that were defined in post-1993 editions of the DICOM standard when converting implicit VR files to explicit VR network transmission. Optional setting, default is: false. |
| ImplicitOnly | true or false | Only negotiate the default Implicit VR Little Endian transfer syntax for all abstract syntaxes. This setting is useful if we're communicating with very old DICOM software which claims to support Explicit VR communication but fails to do so... |
| Supports12Bit | true or false | Accept image pixel data with 12 bits/pixel. Optional setting, default is: true. |
| SupportsPresentationLUT | true or false | Activate support for the Presentation LUT SOP Class. Optional setting, default is: false. |
| PresentationLUTinFilmSession | true or false | The 1999 edition of the DICOM standard contains an inconsistency in the definition of the Presentation LUT SOP class. The attributes "Referenced Presentation LUT", "Illumination" and "Reflected Ambient Light" can either be part of the Basic Film Session or be part of the Basic Film Box. DICOM Correction Proposal CP 173 defines that these attributes have to be specified on Basic Film Box level. However, not all existing Print SCPs supporting the Presentation LUT SOP Class are implemented in accordance with CP 173. The Print SCP can, therefore, be configured to use either Film Session or Film Box (but never both). If flag is true, attributes are accepted and returned in Basic Film Session instead of Basic Film Box. Optional setting, default is: false (which means that behavior is consistent with CP 173.) |
| PresentationLUTMatchRequired | true or false | Some Print SCPs which support Presentation LUTs require that the number of entries in a Presentation LUT matches the bit depth of the image pixel data (4096 entries for 12 bit pixel data, 256 entries for 8 bit pixel data). If flag is true, enforce a matching rule as described above. All Presentation LUT N-CREATE or Basic Grayscale Image Box N-SET operations that would violate the rule will be refused. Optional setting, default is: true. |
| SupportsImageSize | true or false | Some Print SCPs do not support the optional "Requested Image Size" attribute in Basic Grayscale Image Box N-SET operations. If flag is true, support requested image size. Otherwise refuse N-SET operations containing a requested image size attribute. Optional setting, default is: false. |
| SupportsDecimateCrop | true or false | Some Print SCPs do not support the optional "Requested Decimate/Crop Behavior" attribute defined in Supplement 37 in Basic Grayscale Image Box N-SET operations. If flag is true, support requested decimate/crop behavior. Otherwise refuse N-SET operations containing this attribute. Optional setting, default is: false. |
| SupportsTrim | true or false | Some Print SCPs do not support the optional "Trim" attribute in Basic Film Box N-CREATE or N-SET operations. If flag is true, support trim. Otherwise refuse N-CREATE or N-SET operations containing this attribute. Optional setting, default is: false. |

| DisplayFormat | string\string… | this entry specifies all STANDARD\C,R image display formats accepted by the Print SCP for portrait orientation. Landscape image display formats are derived automatically. This is a mandatory entry, no default. |
| --- | --- | --- |
| FilmSizeID | string\string… | Film Size ID identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory setting. |
| MediumType | string\string… | Medium Type identifiers supported by the printer for the Basic Film Session. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| ResolutionID | string\string… | Requested Resolution ID identifiers supported by the printer for the Basic Film Box (optional attribute defined in Supplement 37). Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| MagnificationType | string\string… | Magnification Type identifiers supported by the printer for the Basic Film Box or Basic Grayscale Image Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| SmoothingType | string\string… | Smoothing Type identifiers supported by the printer for the Basic Film Box or Image Box. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| BorderDensity | string\string… | Border Density identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. If any of the identifiers is numeric, then all numbers are accepted. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| EmptyImageDensity | string\string… | Empty Image Density identifiers supported by the printer for the Basic Film Box. Multiple values can be specified, must be separated by '\' characters. If any of the identifiers is numeric, then all numbers are accepted. The first specified value is used as the default. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. |
| MaxDensity | int | Default max density that is used when the Print SCU does not specify max density. This is a mandatory entry. Note: the Print SCP will accept any syntactically correct value for Max Density even if the value is higher than the Max Density default defined here. |
| MinDensity | int | Default min density that is used when the Print SCU does not specify min density. This is an optional entry. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. Note: the Print SCP will accept any syntactically correct value for Min Density even if the value is lower than the Min Density default defined here. |

| Configuration_n | string | Configuration Information that can be sent to the printer for the Basic Film Box or Image Box. Only a single value per entry can be specified because values may contain backslash characters. (VR=ST). Keywords are "Configuration_1", "Configuration_2", etc., without leading zeroes. These are optional settings. If omitted, the Print SCP does not support the attribute and rejects N-CREATE or N-SET requests containing the attribute. The Print SCP default for Configuration Information is always an empty string. |
| --- | --- | --- |
| FilmDestination | string\string… | Film Destination identifiers supported by the Print SCP for the Basic Film Session. Multiple values can be specified, must be separated by '\' characters. The first specified value is used as the default. This is a mandatory entry. |
| OmitSOPClassUIDFromCreateResponse | true or false | The Affected SOP Class UID attribute is optional in DIMSE N-CREATE-RSP messages but some clients rely on its presence. This setting can be used to test client behavior. Defines whether the Print SCP should omit Affected SOP Class UID in DIMSE N-CREATE-RSP messages. This is an optional setting, default is false. |

When the DICOMscope application is started, a separate Print SCP application is started for each section in the configuration file correctly defining a Print SCP.  As an alternative, a Print SCP can also be started manually without the DICOMscope GUI.  The command line Print SCP is called “dcmprscp” and is automatically installed together with DICOMscope.  The following command line options are available:

The name of the configuration file (option –-config) and the printer name (the section heading from the configuration file, option –-printer) should always be passed on the command line.  If the tool is started with the verbose and dump options, a detailed log of it’s operations including a complete log of the DIMSE communication is written to the console.  The –-logfile option causes this information to be written into a text file in the log subdirectory for later examination.

dcmprscp: DICOM basic grayscale print management SCP

usage: dcmprscp [options]

general options:

-h  --help          print this help text and exit

--version       print version information and exit

-v  --verbose       verbose mode, print actions

-d  --debug         debug mode, print debug information

processing options:

-c  --config        [f]ilename: string

process using settings from configuration file

-p  --printer       [n]ame: string (default: 1st printer in cfg file)

select printer with identifier [n] from cfg file

+d  --dump          print all DIMSE messages

-l  --logfile       print to log file instead of stdout

When running, the Print SCP accepts a single DICOM association at a time – multiple clients in parallel are not supported.  Since Print SCP is not connected to a real printer, it does not create hardcopies.  Instead, a print job is stored in the local database (which is shared with the other parts of the DICOMscope application).  Each film of a print job is stored as a DICOM Stored Print object.  Each image of a print job is stored as a DICOM Hardcopy Grayscale Image object.  When Print SCP is run in parallel with the DICOMscope application, the DICOMscope database browser will indicate that new objects have arrived and will allow to load and display these objects.  Hardcopy Grayscale images can be loaded and displayed as any other DICOM image, however, specific characteristics like a Presentation LUT or the image’s polarity are not preserved in the Hardcopy Grayscale image but in the Stored Print object.  Loading a Stored Print causes a preview of the hardcopy to be rendered in DICOMscope’s print preview panel.  This display correctly renders Presentation LUT, Polarity etc.

## Test Procedure

The IHE “Print Request with Presentation LUT“ transaction 23 defines minimum requirements for Print Composer actors.  In particular, the Presentation LUT SOP Class must be supported, and Presentation LUTs must be referenced on Basic Film Box level (in accordance with DICOM Correction Proposal 173).  However, Print Composers still have many “degrees of freedom” in terms of their support for optional attributes.  Therefore, we do not propose a static set of tests for a Print Composer, but instead a (rather general) procedure.  The DICOMscope software default installation defines four Print SCPs:

- IHEFULL accepts associations on port 10005.  This Print SCP supports most optional features of the DICOM print protocol:

- Image display formats 1x1, 1x2, 2x2, 2x3, 3x3, 3x4, 3x5, 4x4 and 4x5 are supported for Portrait orientation, and 1x1, 2x1 etc. for Landscape,

- All standard defined terms for Border Density, Empty Image Density, Film Destination, Film Size ID, Magnification Type, Medium Type and Requested Resolution ID are supported,

- Min Density, Trim, Requested Image Size and Requested Decimate/Crop Behavior are supported,

- Smoothing Type is supported (value: NONE),

- Images can be transmitted with either 8 or 12 bits per pixel,

- The Affected SOP Class UID is always included in N-CREATE-RSP messages,

- Presentation LUTs with a number of entries other than 256 or 4096 are accepted.

- IHERESTRICTED accepts associations on port 10006.  This Print SCP is restricted to the minimum of features required by the IHE technical framework:

- Image display formats must be 1x1

- Border Density, Empty Image Density, Requested Resolution ID, Min Density, Smoothing Type, Trim, Requested Image Size and Requested Decimate/Crop Behavior are not supported,

- None of the standard defined terms for Film Destination and Medium Type are supported

- Only the required terms for Film Size ID and Magnification Type defined in the IHE Technical Framework are supported,

- Images can be transmitted with either 8 or 12 bits per pixel,

- The Affected SOP Class UID is never included in N-CREATE-RSP messages,

- Presentation LUTs with a number of entries other than 256 or 4096 are rejected. Presentation LUTs with 256 entries can only be used with 8 bit/pixel images, LUTs with 4096 entries can only be used with 12 bit/pixel images.

- Only the default Implicit VR Little Endian Transfer Syntax is negotiated.

- IHEFULL_TLS accepts associations on port 10012.  This Print SCP is configured identically to IHEFULL, except that DICOM communication uses TLS (Transport Layer Security).  This SCP can be used with Print Composer actors that also implement the Secure Node actor of the IHE Basic Security Integration Profile.

- IHERESTRICT_TLS accepts associations on port 10013.  This Print SCP is configured identically to IHEFULL, except that DICOM communication uses TLS (Transport Layer Security).  This SCP can be used with Print Composer actors that also implement the Secure Node actor of the IHE Basic Security Integration Profile.

Testing a Print Composer should start with IHEFULL (or its TLS counterpart).  Vendors should create a couple of print jobs exercising the different options available to their Print Composer implementations.  The results of these print jobs should be checked as described in the following two sections.

If the print jobs succeed, the tests should be repeated with IHERESTRICTED (or its TLS counterpart), which poses many restrictions on the Print Composer.  If printing with IHERESTRICTED also succeeds, probability is high that printing on other Print Server actors will also succeed.

A third, optional series of tests can be performed by changing the following setting in the DICOMscope configuration file for IHERESTRICTED and repeating the tests with IHERESTRICTED:

Supports12Bit = false

The DICOMscope software must be restarted after this change.  Now IHERESTRICTED will refuse any image transmitted in a Basic Grayscale Image Box with more than 8 bits/pixel.  This is the most restrictive setting available with the DICOMscope Print SCP.

As described above, all print jobs created by the DICOMscope Print SCP are stored in the local database as a number of Stored Print and Hardcopy Grayscale Image objects.  With a large number of tests, the database can quickly become crowded.  We recommend to erase the database before starting a new series of tests.  This can be done by terminating the DICOMscope application and deleting all files (including index.dat) from the subdirectory named database in the DICOMscope folder.  We also recommend to delete all old log files from the subdirectory named log before starting a new series of tests.

## Syntactical Validation of DICOM Print Communication

The DICOMscope Print SCP creates detailed logs of all DICOM communication with a Print SCU.  The log files are in plain text format and can be opened and printed with any text editor.  They are located in the log subdirectory of the DICOMscope application folder.  The filenames have the form

PrintSCP_XXXX_YYYYMMDD_HHMMSS.log

where XXXX is the symbolic printer name (section title from the DICOMscope configuration file) and YYYYMMDD HHMMSS are date and time when the application was started.  A log file may contain multiple print jobs since it remains open as long as the DICOMscope Print SCP is running.

For each print job, the log file shows the DICOM association negotiation first, followed by the DIMSE messages exchanged between SCU and SCP.  Figure 2.3.1 below shows an example for an association request.  All items of the A-ASSOCIATE-RQ protocol data unit as defined in part 7 and 8 of the DICOM standard are shown.  A basic understanding of the DICOM association protocol (ACSE) is required to read this part of the log.  When evaluating the log, it should be checked whether the Presentation LUT SOP class has been both proposed and accepted.

Figure 2.3.1: Print SCP Log – Association Negotiation

====================== BEGIN A-ASSOCIATE-RQ =====================

Our Implementation Class UID:    1.2.276.0.7230010.3.0.3.4.1

Our Implementation Version Name: OFFIS_DCMTK_341

Their Implementation Class UID:    1.2.276.0.7230010.3.0.3.4.1

Their Implementation Version Name: OFFIS_DCMTK_341

Application Context Name:    1.2.840.10008.3.1.1.1

Calling Application Name:    DCMPSTATE

Called Application Name:     LOCALPRINTER_1

Responding Application Name:

Our Max PDU Receive Size: 32768

Their Max PDU Receive Size: 32768

Presentation Contexts:

Context ID:        1 (Proposed)

Abstract Syntax: =BasicGrayscalePrintManagementMetaSOPClass

Proposed SCP/SCU Role: Default

Accepted SCP/SCU Role: Default

Proposed Transfer Syntax(es):

=BigEndianExplicit

=LittleEndianExplicit

=LittleEndianImplicit

Context ID:        3 (Proposed)

Abstract Syntax: =PresentationLUTSOPClass

Proposed SCP/SCU Role: Default

Accepted SCP/SCU Role: Default

Proposed Transfer Syntax(es):

=BigEndianExplicit

=LittleEndianExplicit

=LittleEndianImplicit

Requested Extended Negotiation: none

Accepted Extended Negotiation: none

======================= END A-ASSOCIATE-RQ ======================

Figure 2.3.2 below shows an example for the DIMSE message log. It shows a N-SET request sent to a Basic Grayscale Image Box object, followed by the N-SET response from the Print SCP.  Evaluating this part of the log requires a basic understanding of the DICOM Message Service Element (DIMSE) and the definition of the DICOM Print Management Service Class.  In particular it should be checked whether a Presentation LUT instance has been created by the Print Composer and whether it has been correctly referenced (activated) as part of the Basic Film Box N-CREATE request.  The status codes sent back by the Print SCP for each message should be checked.  For most error and warning codes returned by the Print SCP, additional information is stored in the log file (preceding the response message).

Figure 2.3.2: Print SCP Log – DIMSE Communication

===================== INCOMING DIMSE MESSAGE ====================

Message Type                  : N-SET RQ

Message ID                    : 5

Requested SOP Class UID       : BasicGrayscaleImageBoxSOPClass

Requested SOP Instance UID    : 1.2.276.0.7230010.3.4.2139363186.2206.960455839.7

Data Set                      : present

# Dicom-Data-Set

# Used TransferSyntax: BigEndianExplicit

(2020,0010) US    1                                      #     2,  1  ImageBoxPosition

(2020,0110) SQ    (Sequence with explicit Length #=1)    #2097262,  1  BasicGrayscaleImageSequence

(fffe,e000) na    (Item with explicit Length  #=9)       #2097254,  1  Item

(0028,0002) US    1                                      #     2,  1  SamplesPerPixel

(0028,0004) CS    [MONOCHROME2]                          #    12,  1  PhotometricInterpretation

(0028,0010) US    1024                                   #     2,  1  Rows

(0028,0011) US    1024                                   #     2,  1  Columns

(0028,0100) US    16                                     #     2,  1  BitsAllocated

(0028,0101) US    12                                     #     2,  1  BitsStored

(0028,0102) US    11                                     #     2,  1  HighBit

(0028,0103) US    0                                      #     2,  1  PixelRepresentation

(7fe0,0010) OW    0160\0160\0160\0160\0177\0177\0177\0177\017d\017d... #2097152,  1  PixelData

(fffe,e00d) na    (ItemDelimitationItem for re-encoding) #     0,  1  ItemDelimitationItem

(fffe,e0dd) na    (SequenceDelimitationItem for re-enc.) #     0,  1  SequenceDelimitationItem

======================= END DIMSE MESSAGE =======================

===================== OUTGOING DIMSE MESSAGE ====================

Message Type                  : N-SET RSP

Message ID Being Responded To : 5

Affected SOP Class UID        : none

Affected SOP Instance UID     : none

Data Set                      : present

DIMSE Status                  : 0x0000: Success

# Dicom-Data-Set

# Used TransferSyntax: UnknownTransferSyntax

(2020,0010) US    1                                      #     2,  1  ImageBoxPosition

======================= END DIMSE MESSAGE =======================

## Visual Validation of DICOM Print Communication

The second step of a print job validation is the visual test.  The stored print object related to the print job should be located in DICOMscope’s database browser and should be loaded into DICOMscope’s Print Preview panel (e. g. by double-clicking on the Stored Print object).  The Print Preview should display a correct softcopy look-alike of the intended hardcopy.  In particular, the images on the Print Preview are displayed with the appropriate Presentation LUT applied.  If DICOMscope is calibrated for the current softcopy display, GSDF is also correctly applied.  It should be noted that certain optional attributes such as Magnification Type, Smoothing Type, Requested Image Size and Requested Decimate/Crop Behavior are ignored in the Print Preview.  Scaling of the images is always done with a bilinear interpolation.  The image data, polarity, Presentation LUT, Image Display Format and Orientation are observed, however.

## Open Issues of DICOM Print and Presentation LUT

This section describes a number of known issues with the DICOM Print Management Service Class that Print Composer vendors should be aware of when testing their implementations.

- The Basic Grayscale Print Meta SOP Class allows image data to be transmitted either with 8 bits/pixel (Bits Stored) or 12 bits/pixel.  There is no requirement, though, that a Print Server must support both.  Vendors might choose to support only one or the other variant.

- The concept of a Presentation LUT is used for two different purposes in the DICOM standard.  On one hand, there is the Presentation LUT SOP Class defined as an extension of the Print Management Service Class.  On the other hand, there is the Softcopy Grayscale Presentation State SOP Class (Supplement 33) which also makes intensive use of Presentation LUTs.  Both SOP Classes make use of the same DICOM attributes and sequences to represent Presentation LUTs.  However, the semantics of how to apply a Presentation LUT to image data are different!  In softcopy, a Presentation LUT with 256 entries can be applied to an image with 12 bits/pixel and result in a correct image display.  This is not allowed in DICOM Print.  There are also differences in the pre-defined Presentation LUT Shapes. “IDENTITY” may be used in Presentation States and in DICOM Print, with different semantics for MONOCHROME1 images, though.  A LUT shape of “INVERSE” is not allowed in DICOM Print while a LUT Shape of “LIN OD” is not allowed in a Softcopy Presentation State. These are inconsistencies in the standard, not in it’s implementation.